IMPORTANT CONCEPTS: AIR QUALITY MANAGEMENT (CET464)

CHAPTER 1: THE ATMOSPHERIC SYSTEM & POLLUTANTS

1.1 The Environmental Framework

The environment operates through four interconnected spheres. A disturbance in one (like the Atmosphere) inevitably affects the others.

- Atmosphere: A protective blanket of gases, mostly Nitrogen (78%) and Oxygen (21%). It regulates Earth's temperature and shields us from UV radiation.

- Hydrosphere: Covers 71% of Earth. Air pollutants like SO₂ can dissolve in water, causing acidification of oceans and lakes.

- Lithosphere: The solid crust. Soil acts as both a source (Radon gas) and a sink (deposition of heavy metals) for air pollution.

- Biosphere: The life zone where plants, animals, and humans reside.

1.2 Defining Air Pollution

Air pollution is the presence of substances (gases, particles, or biological molecules) in the atmosphere that are harmful to health or the environment.

- Natural Sources: Volcanic eruptions (SO₂), forest fires (smoke/CO), and wind-blown dust.

- Anthropogenic Sources: Human-made sources, primarily fossil fuel combustion in vehicles, power plants, and industries.

1.3 Classification of Pollutants

- Primary Pollutants: Emitted directly from a source.

  - Examples: CO from car exhausts, SO₂ from coal power plants.

- Secondary Pollutants: Not emitted directly but formed when primary pollutants react in the atmosphere.

  - Example: Ground-level Ozone (O₃), formed from NOₓ and VOCs reacting in sunlight.

  - Example: Peroxyacetyl Nitrate (PAN), a major component of photochemical smog.

1.4 Criteria Pollutants (The Global Standards)

Governments monitor these specific pollutants to measure air quality:

- Particulate Matter (PM₁₀ and PM₂.₅): Fine dust and liquid droplets.

- Sulfur Dioxide (SO₂): From coal and oil combustion.

- Nitrogen Dioxide (NO₂): From high-temperature combustion in engines.

- Carbon Monoxide (CO): From incomplete combustion.

- Lead (Pb): Historically from leaded petrol; a potent neurotoxin.

- Ozone (O₃): A secondary pollutant in the lower atmosphere.

CHAPTER 2: HISTORICAL LESSONS & RECENT TRENDS

2.1 Classic Air Pollution Episodes

- Meuse Valley (1930): Industrial emissions trapped by a thermal inversion in Belgium; first clear link between industrial air and mass mortality.

- Donora Smog (1948): Similar to Meuse Valley but in the USA. Proved that pollution doesn't just "blow away."

- Great London Smog (1952): A four-day "pea-souper" fog where coal smoke killed 4,000+ people. Led to the UK Clean Air Act of 1956.

2.2 The Indian Scenario: Modern Challenges

- Bhopal Gas Tragedy (1984): While an accident, it remains the turning point for Indian Environmental Law. The resulting Environment Protection Act (1986) is the umbrella under which air quality is managed.

- National Clean Air Programme (NCAP): India’s current strategy to reduce PM levels by 20-30% in non-attainment cities.

- Stubble Burning (Parali): A major seasonal challenge in North India during winter, where crop residue burning in Punjab and Haryana creates severe smog in Delhi.

- BS-VI Transition: India skipped BS-V norms to move directly to BS-VI for vehicles, drastically reducing sulfur and NOₓ emissions.

CHAPTER 3: BIOLOGICAL IMPACTS & HEALTH

3.1 Acute vs. Chronic Effects

- Acute: Immediate responses to high concentrations (e.g., eye stinging, asthma attacks).

- Chronic: Long-term exposure (e.g., lung cancer, COPD, heart disease).

3.2 The "Silent Killer": Carbon Monoxide (CO)

CO is colorless and odorless. Its toxicity lies in its affinity for hemoglobin.

- It forms Carboxyhemoglobin (COHb).

- CO binds 210 times more strongly than oxygen ($O₂$).

- High COHb levels lead to dizziness, unconsciousness, and death by chemical suffocation.

3.3 Fine Particulates (PM₂.₅)

These are dangerous because of their size (< 2.5 micrometers). They bypass the nose/throat filters and enter the alveoli (deep lung sacs), eventually entering the bloodstream and causing systemic inflammation.

CHAPTER 4: VEGETATION & MATERIAL DAMAGE

4.1 Impact on Plants

- Chlorosis: Yellowing of leaves due to chlorophyll loss (common with SO₂ and Fluorides).

- Necrosis: Death of leaf tissue (turning brown or black).

- Abscission: Premature dropping of leaves.

4.2 Material Decay (The Taj Mahal Case)

Acid Rain is the primary culprit. When SO₂ and NO₂ react with water vapor, they form H₂SO₄ and HNO₃.

- Chemical Reaction: $CaCO₃$ (Marble) + $H₂SO₄$ (Acid Rain) → $CaSO₄$ (Gypsum) + $H₂O$ + $CO₂$

- The Gypsum is soft and washes away, a process known as "Stone Leprosy."

CHAPTER 5: INDOOR AIR QUALITY (IAQ)

5.1 The Indoor Risk

Indoor air is often 2–5 times more polluted than outdoor air.

- Radon: A radioactive gas from the soil; the second leading cause of lung cancer.

- VOCs (Volatile Organic Compounds): From paints, furniture (Formaldehyde), and cleaning agents.

5.2 Sick Building Syndrome (SBS)

A condition where occupants feel ill while in a building but feel better once they leave.

- Most Critical Factor: The Air Exchange Rate.

- When ventilation is poor, $CO₂$ levels rise, and toxins build up, leading to headaches and fatigue.

CHAPTER 6: METEOROLOGY & DISPERSION

6.1 Stability and Lapse Rates

- Lapse Rate: The rate at which air temperature decreases with altitude.

- DALR (Dry Adiabatic Lapse Rate): Fixed at 9.8 °C per 1 km.

- ELR (Environmental Lapse Rate): The actual measured temperature change.

6.2 Comparison of Rates

- Unstable Atmosphere (Super-adiabatic): ELR > DALR. Warm air at the bottom rises rapidly. Best for dispersion.

- Stable Atmosphere (Sub-adiabatic): ELR < DALR. Air stays in layers; poor mixing.

- Inversion: The most stable state. Temperature increases with height. Acts as a "lid" that traps pollutants.

6.3 Plume Behavior Patterns

The shape of smoke from a stack tells us about the stability:

- Lofting: Ideal. Inversion below the stack, unstable above. Smoke goes up and stays up.

- Fumigation: Hazardous. Inversion above the stack, unstable below. Smoke is forced down to the ground.

- Fanning: Strong inversion; plume moves horizontally like a thin ribbon.

CHAPTER 7: DISPERSION MODELING

7.1 The Gaussian Plume Model

The standard mathematical tool to predict pollutant concentration ($C$) at any distance downwind.

- Assumptions: Steady emission rate, constant wind speed, and "Normal" (Gaussian) distribution of particles.

- Effective Stack Height (H): H = h + Δh (Physical height + Plume rise). Higher H leads to lower ground-level concentrations.

7.2 Pasquill Stability Categories

Since we cannot always measure ELR, we use six classes (A to F):

- Class A: Very unstable (Strong sun, light wind).

- Class D: Neutral (Cloudy).

- Class F: Very stable (Clear night).

CHAPTER 8: AIR QUALITY MONITORING

8.1 Ambient vs. Stack Sampling

- Ambient: Monitoring the air in the general environment. Station height should be 3–10 meters.

- Stack: Monitoring the "source" (inside the chimney).

8.2 Particulate Sampling (High-Volume Sampler)

- Uses a vacuum to draw air through a glass-fiber filter.

- Operates for 24 hours to give a daily average concentration in $µg/m³$.

8.3 Isokinetic Sampling (Crucial Concept)

In stack sampling, the velocity of the gas entering the sample probe must equal the velocity of the gas moving in the stack.

- If too fast: You miss small particles.

- If too slow: You over-collect large particles.

CHAPTER 9: CONTROL OF PARTICULATES

9.1 Physical Collection Methods

- Settling Chambers: Uses gravity; only for very large particles (> 50 µm).

- Cyclones: Uses centrifugal force. Air spins, and heavy dust hits the walls and falls.

- Fabric Filters (Baghouses): High efficiency for small particles. Works like a giant vacuum filter.

- Electrostatic Precipitators (ESP): Uses high voltage to charge dust particles, which then stick to grounded plates. Most common in Indian power plants.

CHAPTER 10: CONTROL OF GASEOUS POLLUTANTS

10.1 Chemical & Thermal Methods

- Absorption (Scrubbers): Gas is dissolved in a liquid (e.g., using lime spray to remove SO₂).

- Adsorption: Gas molecules "stick" to the surface of a solid like Activated Carbon.

- Combustion/Incineration: Burning VOCs to convert them into less harmful $CO₂$ and $H₂O$.

CHAPTER 11: GLOBAL ATMOSPHERIC ISSUES

11.1 The Greenhouse Effect

Gases like $CO₂$ and $CH₄$ (Methane) allow short-wave solar radiation in but block long-wave terrestrial radiation from escaping, leading to Global Warming.

11.2 Photochemical Smog

A secondary phenomenon common in sunny, traffic-heavy cities (Delhi, LA).

- Mechanism: NOₓ + VOCs + Sunlight → O₃ + PAN.

- Causes a brown haze and severe eye irritation.

CHAPTER 12: INDIAN REGULATORY FRAMEWORK

12.1 The Air Quality Index (AQI)

India’s color-coded system to communicate health risks:

- Good (0–50): Minimal impact.

- Severe (401–500): Affects healthy people and seriously impacts those with existing diseases.

12.2 SAFAR & GRAP

- SAFAR: A system for air quality forecasting in Indian metros.

- GRAP (Graded Response Action Plan): Emergency measures (like banning trucks or construction) that kick in automatically when the AQI crosses certain thresholds in the Delhi-NCR region.