MODULE 2: EFFECTS OF AIR POLLUTION & INDOOR AIR QUALITY

Course: CET464 – Air Quality Management

2.1 Impact on Human Health: Acute vs. Chronic

The physiological response of the human body to air pollutants depends on the dosage, which is a function of concentration and duration of exposure.

- Acute Effects: These are immediate, short-term responses to high concentrations of pollutants. They often occur during "pollution episodes" (like the London Smog). Symptoms include sudden breathlessness, severe eye irritation, or a sudden asthmatic attack.

- Chronic Effects: These are more insidious and result from long-term exposure to lower levels of pollutants. The damage accumulates over years, often leading to irreversible conditions such as lung cancer, emphysema, or permanent reduction in lung capacity.

Specific Biological Mechanisms

- The Respiratory System: Pollutants like SOₓ and NOₓ are "respiratory irritants." When inhaled, they react with the moisture in the respiratory tract to form acidic compounds that inflame the lining of the lungs. This leads to bronchoconstriction (narrowing of airways), making it difficult to breathe.

- The Cardiovascular System (Carbon Monoxide): CO is particularly dangerous because it is chemically "sticky." It binds to hemoglobin to form Carboxyhemoglobin (COHb). Since CO binds 210 times more strongly than oxygen, it effectively "starves" the heart and brain of oxygen. High CO levels can lead to myocardial infarction (heart attack) or unconsciousness.

- Particulate Matter (The Size Factor): * PM₁₀: Mostly trapped in the cilia and mucus of the upper respiratory tract.

  - PM₂.₅: These "fine" particles bypass the body's natural filters and settle in the alveoli (air sacs), where gas exchange occurs. This can lead to systemic inflammation.

  - Ultrafine Particles (<100 nm): These are small enough to pass through the alveolar wall directly into the bloodstream, potentially damaging the liver, kidneys, and even the brain.

2.2 Effects on Vegetation and the Environment

Plants are highly sensitive "bio-indicators" of air quality. Pollutants enter through the stomata—the microscopic pores on leaves used for photosynthesis and respiration.

- Chlorosis: The most common symptom, where the plant cannot produce enough chlorophyll. Leaves turn pale or yellow, drastically reducing the plant's ability to create food through photosynthesis.

- Necrosis: This is the actual death of plant tissue. It appears as brown or dried-out spots on the leaf surface, often between the veins (interveinal necrosis).

- Epinasty and Abscission: Some pollutants (like ethylene) cause leaves to curl downward (epinasty) or fall off the plant prematurely (abscission), leading to stunted growth or crop failure.

The Threat of Acid Rain

Acid rain ($H₂SO₄$ and $HNO₃$) forms when SO₂ and NOₓ react with atmospheric water vapor. It lowers the pH of soil and water bodies.

- Soil Impact: It leaches out essential nutrients like Calcium and Magnesium while releasing toxic Aluminum from the soil, which kills trees and crops.

- Aquatic Impact: Most fish and aquatic organisms cannot survive in water with a pH below 5.0, leading to "dead lakes."

2.3 Effects on Materials and Infrastructure

Air pollution causes the "deterioration of materials," leading to billions of dollars in economic loss annually.

- Corrosion of Metals: SO₂ and moisture form sulfuric acid, which accelerates the rusting of steel and the corrosion of copper and aluminum.

- Damage to Stone (The "Stone Leprosy"): Carbonate rocks like Marble and Limestone (e.g., the Taj Mahal) are dissolved by acid rain. The Calcium Carbonate ($CaCO₃$) reacts with acid to form Gypsum ($CaSO₄$), which is soft and easily washed away by rain, blurring the fine details of sculptures and buildings.

- Tarnishing: Hydrogen Sulfide ($H₂S$) reacts with silver and lead-based paints, causing them to turn black or grey.

2.4 Indoor Air Quality (IAQ) and Sick Building Syndrome

Indoor air is often more polluted than outdoor air because of a lack of Air Exchange.

Common Indoor Pollutants

- Radon Gas: A naturally occurring radioactive gas that seeps from the soil into basements. It is the second leading cause of lung cancer.

- Volatile Organic Compounds (VOCs): Emitted as gases from certain solids or liquids. Common sources include paints, varnishes, cleaning supplies, and "new furniture" smell (Formaldehyde).

- Biological Contaminants: Mold, mildew, and dust mites thrive in high-humidity environments.

Sick Building Syndrome (SBS)

SBS describes a situation where building occupants experience acute health effects that appear to be linked to time spent in a building, but no specific illness can be identified.

- Key Symptoms: Headache, dry cough, dizziness, nausea, and difficulty concentrating.

- Critical Factor (Ventilation): The most common cause is poor ventilation. When a building's Air Exchange Rate is too low, $CO₂$ and other toxins build up. A healthy building should replace its entire volume of air multiple times per hour.