MODULE 4: AIR QUALITY MONITORING & SAMPLING

Course: CET464 – Air Quality Management

Scope: Ambient vs. Stack Sampling, Measurement Techniques, and Emission Inventories.

4.1 Introduction to Air Quality Monitoring

Air Quality Monitoring is the systematic, long-term assessment of pollutant levels by measuring the quantity and types of certain pollutants in the surrounding, outdoor air. It serves as the foundation for all pollution control programs.

Objectives of Monitoring

- Compliance: To determine if the air quality meets the National Ambient Air Quality Standards (NAAQS).

- Trend Analysis: To evaluate if air quality is improving or deteriorating over time.

- Policy Evaluation: To assess whether existing control strategies (like vehicle emission norms) are actually working.

- Health Warning Systems: To provide data for Air Quality Index (AQI) reports that warn sensitive groups (children/elderly) during high pollution days.

4.2 Types of Sampling

Measurement generally falls into two distinct categories based on where the sample is taken:

1. Ambient Air Sampling

This measures the air we breathe in our general surroundings (residential areas, city centers, parks). The goal is to obtain a "representative sample" of the air in a specific region.

- Station Placement: Samplers must be placed away from immediate obstructions like tall buildings or trees that might block wind flow.

- Height: Usually placed 3 to 10 meters above ground level to capture the air at the human breathing zone while avoiding heavy surface dust.

2. Stack Sampling (Source Sampling)

This deals with pollutants emitted directly from a specific source, such as a factory chimney or an exhaust pipe.

- Purpose: To check if a factory is staying within its legal emission limits.

- Isokinetic Sampling: This is a critical technical requirement for stack sampling. It means that the speed at which the air enters the sampling nozzle must be exactly the same as the speed of the gas moving inside the stack. If the speeds differ, the sample will not accurately represent the concentration of heavy vs. light particles.

4.3 Sampling of Particulate Matter

Since particles vary in size, different techniques are used to capture them:

- Sedimentation (Dust Fall Jar): The simplest method. A jar is left open for 30 days, and the "settleable" dust is weighed. This only captures very large particles.

- Filtration (High-Volume Sampler): This is the most common method for PM₁₀ and PM₂.₅. A large volume of air is pulled through a pre-weighed filter (usually glass fiber). After 24 hours, the filter is weighed again. The weight gain divided by the air volume gives the concentration in $µg/m³$.

- Impaction: Air is directed at a high speed toward a surface; particles hit the surface and stick to it. Cascade impactors use this to separate particles by size.

4.4 Sampling of Gaseous Pollutants

Gases cannot be caught on a simple filter, so they must be trapped using chemical or physical properties:

- Absorption (Bubblers/Impingers): The air is bubbled through a liquid chemical that reacts with and "traps" the gas. For example, SO₂ is often absorbed in a solution of sodium tetrachloromercurate.

- Adsorption: The gas molecules stick to the surface of a solid (like activated carbon or silica gel).

- Grab Sampling: A sample of air is physically trapped in a specialized bag (Tedlar bag) or an evacuated canister and taken to a laboratory for analysis.

4.5 Emission Inventory

An Emission Inventory is a comprehensive database that lists the amount of air pollutants discharged into the atmosphere from all sources in a specific geographical area during a specific time period.

Importance of the Inventory

- Future Projections: By looking at current data and population growth, engineers can predict what the air quality will be like in 10 years.

- Identifying Major Contributors: It helps determine if the majority of a city's pollution comes from cars, industries, or household cooking.

- Cost-Effective Strategy: If the inventory shows that 80% of SO₂ comes from one specific power plant, authorities can focus their budget on fixing that one source rather than across the whole city.