TUTORIAL SHEET 1: MODULE 1 & 2

Subject: CET464 – Air Quality Management

Part A: Fundamental Concepts

- Define Environment based on the ISO definition and explain the interrelationship between the four spheres of the earth.

- Differentiate between Primary and Secondary pollutants. Provide the chemical formation process for Ground-level Ozone (O₃).

- Explain the "Silent Killer" mechanism of Carbon Monoxide (CO) and its affinity for hemoglobin.

- List the historical significance of the 1952 Great London Smog. How did it change environmental policy?

- Define PM₂.₅ and PM₁₀. Explain why finer particles pose a greater risk to the human cardiovascular system.

Part B: Effects and Indoor Air Quality

- Vegetation Impact: Define and distinguish between Chlorosis, Necrosis, and Abscission in plants.

- Material Damage: Explain the chemical reaction between Acid Rain (H₂SO₄) and Calcium Carbonate (CaCO₃) in marble monuments.

- Indoor Health: Discuss the sources of Radon gas and Volatile Organic Compounds (VOCs) in modern office buildings.

- Sick Building Syndrome: Identify the most critical factor governing SBS and suggest how the Air Exchange Rate affects occupant health.

TUTORIAL SHEET 2: MODULE 3

Subject: CET464 – Air Quality Management

Part A: Atmospheric Physics

- Define the Environmental Lapse Rate (ELR) and the Dry Adiabatic Lapse Rate (DALR). State the standard value for DALR.

- Explain Atmospheric Inversion. Why does it lead to a "Cap" or "Lid" effect over industrial cities?

- Describe the function of a Wind Rose. How is the "prevailing wind direction" determined from this diagram?

- List the four major assumptions required for the Gaussian Plume Model to be valid.

Part B: Stability and Dispersion

- Stability Calculation: If the temperature at ground level is 28 °C and the temperature at an altitude of 1.5 km is 10 °C, calculate the ELR and determine if the atmosphere is Stable or Unstable.

- Plume Analysis: Sketch and explain the meteorological conditions required for:

  - Lofting (The most desirable plume)

  - Fumigation (The most hazardous plume)

- Pasquill Categories: Compare Stability Category A (Strongly Unstable) and Stability Category F (Moderately Stable) in terms of solar radiation and wind speed.

TUTORIAL SHEET 3: MODULE 4

Subject: CET464 – Air Quality Management

Part A: Sampling Principles

- Distinguish between Ambient Air Quality Monitoring and Stack Sampling.

- Explain the necessity of Isokinetic Sampling when measuring particulate matter in moving gas streams. What happens to the sample if the nozzle velocity is too high?

- What is an Emission Inventory? How does it assist in cost-effective air quality policy planning?

- Describe the working principle of a High-Volume Sampler (HVS) for measuring PM₁₀.

Part B: Technical Methods

- Gas Capture: Explain the processes of Absorption (using Impingers) and Adsorption (using surface media) for gaseous pollutant collection.

- Particle Separation: Describe how a Cascade Impactor uses the principle of impaction to separate particles by their aerodynamic diameter.

- Site Selection: You are tasked with placing an ambient monitoring station in an urban area. Justify the requirements for sampler height and distance from the nearest roadway.

Instructions for Students:

- Use appropriate chemical notations (e.g., SO₂, NOₓ, H₂SO₄) in all descriptive answers.

- Diagrams for Plume Behavior and the Wind Rose must be labeled accurately.

- Calculations must clearly show the comparison between ELR and DALR (9.8 °C/km).